B1	B2	B15

命令	有效载荷字节，14个字节

令：取值范围0x000x7F; Bit7永远为0

后一个字节是CRC，CRC的计算就是简单把前边所有15个字节的相加，然后取最低8位，例如如果数据包为：A1 A2 A3 A4	A13 A14

C = (A1 + A2 + A3 + A4+……+ A13 + A14 + A15) & 0xFF；

能戒指收到命令后返回16个字节的应答数据包，格式如下：

B1	B2	B15

|  | 文件名称 File Name |  | 蓝牙穿戴产品API协议（保密） Bluetooth Wearable Product API Protocol (Confidential) |  | 最新版本 Latest Version |  |  | V1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 产品编号 Product Number |  | JCV8 | 产品名称 Product Name | 蓝牙智能无屏手环 Smart screenless wristband | 发布日期 Release Date |  | 2026年1月26日 Jan 26, 2026 |  |
| 一、蓝牙配置描述 |  |  |  |  |  |  |  |  |
| 1 | Service UUID |  |  | 0xFFF0 |  |  |  |  |
| 2 | 特征UUID |  | TX UUID | 0xFFF6 APP发送数据给戒指，长度为16字节 |  |  |  |  |
|  |  |  | RX UUID | 0xFFF7 戒指发送数据给APP，长度为16字节 |  |  |  |  |
| 3 | 备注 |  |  | Bluetooth通信只是在两个特征UUID间进行交互，具体协议格式见第二章。 |  |  |  |  |
| 二、蓝牙协议 |  |  |  |  |  |  |  |  |
| 1 | APP发送16个字节的数据包给智能戒指，然后智能戒指发送返回16个字节的数据包。除了个别命令外都利用这种通信方式. |  |  |  |  |  |  |  |
| 2 | APP发送下去的16个字节的数据包格式如下： |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | B16 |  |
|  |  |  |  |  |  |  | CRC校验字节 |  |
|  | 命 |  |  |  |  |  |  |  |
|  | 最 CR |  |  | A15 CRC，则 |  |  |  |  |
| 3 | 智 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | B16 |  |
|  | 命令 |  | 有效载荷字节，14个字节 |  |  |  | CRC校验字节 |  |
|  | 命令：回复收到的命令. |  |  |  |  |  |  |  |
|  | 如收到0x01命令，且CRC检验OK，则返回0x01，如果CRC出错则将Bit7置1，返回0x81. |  |  |  |  |  |  |  |
|  | 最后一个字节是CRC，CRC的计算就是简单把前边所有15个字节的相加，然后取最低8位，例如如果数据包为：A1 A2 A3 A4 A13 A14 A15 CRC，则 CRC = (A1 + A2 + A3 + A4+……+ A13 + A14 + A15) & 0xFF； |  |  |  |  |  |  |  |
| Ⅰ、Bluetooth Profile Description |  |  |  |  |  |  |  |  |
| 1 | Service UUID |  |  | 0xFFF0 |  |  |  |  |
|  |  |  | TX UUID | 0xFFF6 The app sends data to the ring, with a length of 16 bytes. |  |  |  |  |

密有保所！件深圳市友宏2科技有限公司版权Characteristic U文UID ，请严格遵守保密协议，请勿外传给第三方！	1

|  |  | RX UUID | 0xFFF7 The ring sends data to the app, with a length of 16 bytes. |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | Remark |  | Bluetooth communication only interacts between two characteristic UUIDs. The specific protocol format is described in Chapter 2. |  |  |
| Ⅱ、Bluetooth Protocol |  |  |  |  |  |
| 1 | The app sends a 16-byte data packet to the smart ring, and then the smart ring sends back a 16-byte response packet. This communication method is used for all commands except a few specific ones. |  |  |  |  |
| 2 | The |  |  |  |  |
|  |  |  |  |  | B16 |
|  |  |  |  |  | CRC Checksum Byte |
|  | Co |  |  |  |  |
|  | The ket is: A1 A2 A3 A4... A13 A14 A15 CRC, then CRC = (A1 + A2 |  |  |  |  |
| 3 | When the smart ring receives a command, it responds with a 16-byte reply data packet. The format is as follows: |  |  |  |  |
|  | B1 | B2 B15 |  |  | B16 |
|  | Command | Payload bytes, 14 bytes |  |  | CRC Checksum Byte |
|  | Command: Reply with the received command. |  |  |  |  |
|  | If the 0x01 command is received and the CRC check is OK, return 0x01. If the CRC is incorrect, set Bit 7 to 1 and return 0x81. |  |  |  |  |
|  | The last byte is the CRC. The CRC calculation is simply the sum of all the previous 15 bytes, then taking the lowest 8 bits. For example, if the data packet is: A1 A2 A3 A4... A13 A14 A15 CRC, then CRC = (A1 + A2 + A3 + A4 + ... + A13 + A14 + A15) & 0xFF; |  |  |  |  |
| API协议--文件修改记录 API Protocol - Document Revision History |  |  |  |  |  |
| 序号 Serial Number | 版本 Version | 修改内容 Content that has been modified | 修改人 Modifier | 更新日期 Update Date | 批准人/日期 |

format of the 16-byte data packet sent by the app is as follows:

B1	B2	B15

Command	Payload bytes, 14 bytes

mmand: Value range 0x00 to 0x7F; Bit 7 is always 0.

last byte is the CRC. The CRC calculation is simply the sum of all the previous 15 bytes, then taking the lowest 8 bits. For example, if the data pac

+ A3 + A4 +	+ A13 + A14 + A15) & 0xFF;

| 1 | V1.0 | 初始版本 Initial Version | 周胜 | 2025年7月18日 |  |
| --- | --- | --- | --- | --- | --- |

Command Format: 0x05 ID5 ID4 ID3 ID2 ID1 ID0 00 00 00 00 00 00 00 00 CRC

00 00 00 00 CRC	Function: The APP writes the ID code to the bracelet.

Description: The 6-byte ID code is arranged with the high byte first. Command Response:

If CRC is correct and execution is OK:

0 00 00 00 00 00 00 CRC	0x05 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

0 00 00 00 00 00 00 CRC	If CRC is incorrect or execution fails:

0x85 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Command Format: 0x13 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC

Function: Read the battery level of the ring.

00 00 CRC	Description:

Command Response:

If CRC is correct and execution is OK:

0x13 AA BB CC DD EE FF GG 00 00 00 00 00 00 00 CRC

GG 00 00 00 00 00 00 00 CRC	AA: Battery level(in hexadecimal), value from 0 to 100, indicating the battery percentage.

Command format: 0x12 00 00 00 00 00 00 00 00 00

AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC	Function: Restore to factory minimum current.

，0：恢复到出厂最小电流	Description: Puts the device into sleep mode. The sl

休眠模式 ，休眠模式需要充电激活后才能正常使用	Command Response:

If the checksum is correct and execution is successf

K返回：0x12 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC	0x12 00 00 00 00 00 00 00 00 00 00 00 00 00 00 C

l返回：0x92 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC	If there is a checksum error or execution fails:

0x92 00 00 00 00 00 00 00 00 00 00 00 00 00 00 C

Command format: 0x2E 00 00 00 00 00 00 00 00 00

0 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC	Function: Reset the main control MCU.

CU	Description: Restart the main control unit (MCU) an

重置测量模式	Command Response:

If the checksum is correct and execution is successf

K返回：0x2E 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC	0x2E 00 00 00 00 00 00 00 00 00 00 00 00 00 00 C

l返回：0xAE 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC	If there is a checksum error or execution fails:

0xAE 00 00 00 00 00 00 00 00 00 00 00 00 00 00 C

| 深圳市友宏科技有限公司 （保密） Shenzhen Youhong Technology Co., Ltd. (Confidential) . |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 蓝牙穿戴产品 API 协议 产品名称：Smart screenless wristband 型号：JCV8 Bluetooth Wearable Product API Protocol. Product Name:Smart screenless wristband. Model: JCV8 |  |  |  |  |  |  |  |  |
| 序号 Serial Number |  | 功能描述 Function Description | 命令格式描述 |  | Command format description | 适配产品类型 Compatible product types | 分类 Category | 备注 Remarks |
|  |  |  | 命令格式： 0x01 AA BB CC DD EE FF 00 00 00 00 00 00 00 00 CRC 功能：写入APP当前时间到戒指 ，共16bytes( 时间参数以十进制输入 ) 描述： AA:年 年份参数取值范围 ： 99>=年份>=0。如12年，AA = 12（默认为21世纪，即20AA年) BB:月 月份参数取值范围 ： 12>=月份>=1 CC:日 日期参数应在对应的月份实际天数内 ，如一月则参数取值范围为 1-31 DD:小时 小时参数取值范围 ： 23>=小时>=0 EE:分钟 分钟参数取值范围 ： 59>=分钟>=0 FF:秒 秒钟参数取值范围 ： 59>=秒钟>=0 命令回复： 校验正确且执行OK返回： 0x01 F4 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 校验错误或执行Fail返回：0x81 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 例如：发送 0x01 24 08 08 08 08 08 00 00 00 00 00 00 00 00 CRC 返回 0x01 F4 00 00 00 00 00 00 00 00 00 00 00 00 00 F5 F4表示蓝牙数据包最大长度 （即蓝牙所允许单次发送数据的最大长度 ） |  | Command Format: 0x01 AA BB CC DD EE FF 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Function: Writes the current time from the APP to the ring, 16 bytes (time parameters are entered in decimal) |  |  |  |
|  |  |  |  |  | Description: |  |  |  |
|  |  |  |  |  | AA: Year (Year parameter range: 99 ≥ Year ≥ 0. For example, for 2012, AA = 12, default is 21st century, so AA = 12 means the year 2012). |  |  |  |
|  |  |  |  |  | BB: Month (Month parameter range: 1 ≤ Month ≤ 12). |  |  |  |
|  |  |  |  |  | CC: Day (Day parameter should be within the actual number of days in the corresponding month. For example, for January, valid values are from 1 to 31). |  |  |  |
|  |  |  |  |  | DD: Hour (Hour parameter range: 0 ≤ Hour ≤ 23). |  |  |  |
|  |  |  |  |  | EE: Minute (Minute parameter range: 0 ≤ Minute ≤ 59). |  |  |  |
| 1 |  | 设置时间 Set Time |  |  | FF: Second (Second parameter range: 0 ≤ Second ≤ 59). Response: Successful Response (valid checksum and success): | JCV8 | 通用设置 General Settings |  |
|  |  |  |  |  | 0x01 F4 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Failure Response (checksum error or failure in execution): |  |  |  |
|  |  |  |  |  | 0x81 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Example: |  |  |  |
|  |  |  |  |  | Command Sent: 0x01 24 08 08 08 08 08 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | This means the year is 2012, the month is August, the day is 8th, hour is 8, minute is 8, and second is 8. |  |  |  |
|  |  |  |  |  | Response Received: 0x01 F4 00 00 00 00 00 00 00 00 00 00 00 00 00 F5 |  |  |  |
|  |  |  |  |  | F4 indicates the maximum Bluetooth data packet length (the maximum data that Bluetooth can send in a single transmission). |  |  |  |
|  |  |  | 命令格式：0x41 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 功能： APP从戒指上取得当前时间 ，共16bytes（返回命令中时间参数为十进制 ） 校验正确且执行OK返回：0x41 AA BB CC DD EE FF GG WW 00 00 00 00 00 00 CRC 校验错误或执行Fail返回：0xC1 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 回复描述： AA BB CC DD EE FF ：年月日时分秒，如12年，AA = 0x12（默认为21世纪，即20AA年） GG：星期,01-07 代表周一到周日 WW：蓝牙数据包最大长度 （默认F4，代表整个命令最大长度 ）命令回复： 例如：发送 0x41 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x41 24 08 08 08 10 47 06 F4 00 00 00 00 00 00 CRC 24：2024年¦08：8月¦08：8日¦8：8点¦10：01分¦06：星期六¦F4：蓝牙数据包最大长度 （即蓝牙通道所允许单次发送 244个字节数据） |  | Command Format: 0x41 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Function: The APP retrieves the current time from the ring, a total of 16 bytes (time parameters in the response are in decimal format). |  |  |  |
|  |  |  |  |  | Response: |  |  |  |
|  |  |  |  |  | Successful Response (valid checksum and success): |  |  |  |
|  |  |  |  |  | 0x41 AA BB CC DD EE FF GG WW 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Failure Response (checksum error or failure in execution): |  |  |  |
|  |  |  |  |  | 0xC1 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Response Description: |  |  |  |
|  |  |  |  |  | AA: Year (e.g., 12 means 2012, default for the 21st century is 20AA year). |  |  |  |
|  |  |  |  |  | BB: Month (1 to 12). |  |  |  |
|  |  |  |  |  | CC: Day (1 to 31, depending on the month). |  |  |  |
|  |  |  |  |  | DD: Hour (0 to 23). |  |  |  |
|  |  |  |  |  | EE: Minute (0 to 59). |  |  |  |
| 2 |  | 取得时间 Get time |  |  | FF: Second (0 to 59). GG: Weekday (1-7, where 01 is Monday and 07 is Sunday). | JCV8 | 通用设置 General Settings |  |
|  |  |  |  |  | WW: Bluetooth data packet maximum length (default is F4, indicating the Bluetooth channel allows sending 244 bytes of data per transmission). |  |  |  |
|  |  |  |  |  | Example: |  |  |  |
|  |  |  |  |  | Command Sent: 0x41 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | The APP requests the current time from the ring. |  |  |  |
|  |  |  |  |  | Response Received: 0x41 24 08 08 08 10 47 06 F4 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | 24: Year 2024 |  |  |  |
|  |  |  |  |  | 08: Month 8 (August) |  |  |  |
|  |  |  |  |  | 08: Day 8 |  |  |  |
|  |  |  |  |  | 08: Hour 8 |  |  |  |
|  |  |  |  |  | 10: Minute 01 |  |  |  |
|  |  |  |  |  | 47: Second 47 |  |  |  |
|  |  |  |  |  | 06: Weekday (Saturday) |  |  |  |
|  |  |  |  |  | F4: Bluetooth data packet maximum length (244 bytes) |  |  |  |
|  |  |  |  |  | Command Format: 0x02 AA BB CC DD EE 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Function: Write personal information to the ring (input parameters are in hexadecimal format). |  |  |  |
|  |  |  |  |  | Description: |  |  |  |
|  |  |  |  |  | AA: Gender (0 for female, 1 for male). |  |  |  |
| 3 |  | 设置用户个人信息 Set user personal information | 命令格式： 0x02 AA BB CC DD EE 00 00 00 00 00 00 00 00 00 CRC 功能：向戒指写入个人信息 （输入的个人信息参数为 16进制）描述：AA: 性别（0表示女性，1表示男性） BB:年龄（岁，整数） CC:身高（cm，精度1cm） DD:体重（kg，精度1kg） 命令回复： 校验正确且执行OK返回：0x02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 校验错误或执行Fail返回：0x82 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 例如：发送 0x02 00 25 AA 32 3B 00 00 00 00 00 00 00 00 00 CRC 返回 0x02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  | BB: Age (in years, integer). CC: Height (in cm, precision of 1 cm). DD: Weight (in kg, precision of 1 kg). Response: Successful Response (valid checksum and success): 0x02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Failure Response (checksum error or failure in execution): 0x82 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Example: Command Sent: 0x02 00 25 AA 32 3B 00 00 00 00 00 00 00 00 00 CRC AA = 00 (Female) BB = 25 (Age 25) | JCV8 | 健康设置 Health Settings |  |
|  |  |  |  |  | CC = AA (Height 170 cm in hex) |  |  |  |
|  |  |  |  |  | DD = 32 (Weight 50 kg in hex) |  |  |  |
|  |  |  |  |  | Response Received: 0x02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | A successful response indicating the personal information was written to the ring. |  |  |  |
|  |  |  | 命令格式： 0x42 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 功能：APP从戒指中读取个人信息命令回复： 校验正确且执行OK返回：0x42 AA BB CC DD EE FF GG HH II JJ KK 00 00 00 CRC 校验错误或执行Fail返回：0xC2 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 回复描述：AA:性别（0表示女性，1表示男性） BB:年龄（岁，整数，16进制） CC:身高（cm，精度1cm，16进制） DD:体重（kg，精度1kg，16进制） EE:步长（cm，16进制） FF - KK为6字节的戒指ID码，大端 例如：发送 0x42 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x42 00 25 AA 32 3B 30 30 30 30 30 30 00 00 00 CRC 00：性别女性¦25：年龄37岁¦AA：身高170cm¦32 ：体重50kg¦3B：步长59cm¦30 30 30 30 30 30：戒指ID码，大端 |  | Command Format: 0x42 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Function: The app reads personal information from the ring. |  |  |  |
|  |  |  |  |  | Response: |  |  |  |
|  |  |  |  |  | Successful Response (valid checksum and success): |  |  |  |
|  |  |  |  |  | 0x42 AA BB CC DD EE FF GG HH II JJ KK 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Failure Response (checksum error or failure in execution): |  |  |  |
|  |  |  |  |  | 0xC2 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Response Description: |  |  |  |
|  |  |  |  |  | AA: Gender (0 for female, 1 for male) |  |  |  |
|  |  |  |  |  | BB: Age (in years, hexadecimal) |  |  |  |
| 4 |  | 读取用户个人信息 Get user personal information |  |  | CC: Height (in cm, with a precision of 1 cm, hexadecimal) DD: Weight (in kg, with a precision of 1 kg, hexadecimal) EE: Step length (in cm, hexadecimal) | JCV8 | 健康设置 Health Settings |  |
|  |  |  |  |  | FF - KK: 6-byte ring ID code (big-endian) |  |  |  |
|  |  |  |  |  | Example: |  |  |  |
|  |  |  |  |  | Command Sent: 0x42 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Response Received: 0x42 00 25 AA 32 3B 30 30 30 30 30 30 00 00 00 CRC |  |  |  |
|  |  |  |  |  | 00: Female (Gender) |  |  |  |
|  |  |  |  |  | 25: 37 years old (Age) |  |  |  |
|  |  |  |  |  | AA: 170 cm (Height) |  |  |  |
|  |  |  |  |  | 32: 50 kg (Weight) |  |  |  |
|  |  |  |  |  | 3B: 59 cm (Step length) |  |  |  |
|  |  |  |  |  | 30 30 30 30 30 30: Ring ID (Big-endian representation) |  |  |  |
|  |  |  | 命令格式：0x05 ID5 ID4 ID3 ID2 ID1 ID0 00 00 00 00 |  |  |  |  |  |
|  |  |  | 功能：APP写入ID码到手环中； |  |  |  |  |  |
| 5 |  | 设置戒指ID码 Set ring ID | 描述：6个Bytes的ID码排列是高字节在前命令回复： |  |  | JCV8 | 通用设置 General Settings |  |
|  |  |  | 校验正确且执行OK返回：0x05 00 00 00 00 00 00 00 0 |  |  |  |  |  |
|  |  |  | 校验错误或执行Fail返回：0x85 00 00 00 00 00 00 00 0 |  |  |  |  |  |
|  |  |  | 命令格式： 0x13 00 00 00 00 00 00 00 00 00 00 00 00 |  |  |  |  |  |
|  |  |  | 功能：读取戒指的电量 |  |  |  |  |  |
| 6 |  | 读取戒指电量 Read ring battery level | 描述： 命令回复： 校验正确且执行OK返回：0x13 AA BB CC DD EE FF |  |  | JCV8 | 通用设置 General Settings |  |
|  |  |  | AA 电量级别(16进制)，取值为0 to 100, 表示的电量百分比。BB 充电状态，1: 充电中 0：没有充电;CC DD：电压值，十进制 |  | BB: Charging status, 1 means charging, 0 means not charging. |  |  |  |
|  |  |  | 校验错误或执行Fail返回：0x93 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  | CC DD: Voltage value, in decimal format. |  |  |  |
|  |  |  |  |  | If CRC is incorrect or execution fails: |  |  |  |
|  |  |  |  |  | 0x93 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Command format: 0x22 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  | 命令格式： 0x22 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  | Function: Read the MAC address of the ring. |  |  |  |
|  |  |  | 功能：读取戒指的MAC地址。 |  | Description: |  |  |  |
| 7 |  | 读取MAC地址 Read MAC address | 描述： 命令回复： |  | Command Response: If the checksum is correct and execution is successful: | JCV8 | 通用设置 General Settings |  |
|  |  |  | 校验正确且执行OK返回：0x22 MAC0 MAC1 MAC2 MAC3 MAC4 MAC5 00 00 00 00 00 00 00 00 CRC |  | 0x22 MAC0 MAC1 MAC2 MAC3 MAC4 MAC5 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  | 校验错误或执行Fail返回：0xA2 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  | If there is a checksum error or execution fails: |  |  |  |
|  |  |  |  |  | 0xA2 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Command format: 0x27 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Function: Read the software version number. |  |  |  |
|  |  |  | 命令格式： 0x27 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  | Command Response: |  |  |  |
|  |  |  | 功能：读取软件版本号 。 |  | If the checksum is correct and execution is successful: |  |  |  |
| 8 |  | 读取固件版本号 Read firmware version | 命令回复： 校验正确且执行OK返回：0x27 AA BB CC DD EE FF GG 00 00 00 00 00 ID CRC |  | 0x27 AA BB CC DD EE FF GG 00 00 00 00 00 ID CRC If there is a checksum error or execution fails: | JCV8 | 通用设置 General Settings |  |
|  |  |  | 校验错误或执行Fail返回：0xA7 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  | 0xA7 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  | 回复描述：表示为AA BB CC DD为软件版本号（十六进制BCD码格式），EE FF GG 为对应时间年月日 |  | Response Description: |  |  |  |
|  |  |  |  |  | AA BB CC DD represent the software version number (in hexadecimal BCD format). |  |  |  |
|  |  |  |  |  | EE FF GG represent the corresponding time in year, month, and day. |  |  |  |
| 9 |  | 恢复出厂设置 Restore factory settings | 命令格式： 0x12 功能：AA:1, 关机描述：将设备进入命令回复： 校验正确且执行O 校验错误或执行Fai |  | 00 00 00 00 00 CRC eep mode requires charging to activate before it can be used normally. ul: RC RC | JCV8 | 通用设置 General Settings |  |
| 10 |  | MCU软复位 MCU soft reset | 命令格式： 0x2E 0功能：复位主控M描述：重启主控，命令回复： 校验正确且执行O 校验错误或执行Fai |  | 00 00 00 00 00 CRC d reset the measurement mode. ul: RC RC | JCV8 | 通用设置 General Settings |  |
|  |  |  | 命令格式： 0x09 AA BB 00 00 00 00 00 00 00 00 00 00 00 00 CRC 功能：启动或关闭实时计步模式 AA： 1:启动实时计步 0;关闭实时计步 BB： 1:开启实时温度 0;关闭实时温度命令回复： 校验正确且执行OK返回：0x09 AA BB CC DD EE FF GG HH II JJ KK LL 00 00 00 00 00 00 00 00 UU T1 T2 S1 00(无CRC校验，返回31个字节) 回复描述： 启动成功后，戒指数据有变化时 ，将会自动重传命令回复 （下列数据字节默认为 16进制小端） AA BB CC DD：总步数 EE FF GG HH：卡路里值（卡路里显示时请除以 100，保留两位小数，单位是KCAL） II JJ KK LL：步行距离（距离显示时请除以 100 ，保留两位小数，单位是KM） UU： 心率值（大端） T1 T2 ： 温度变化值 （精度0.1℃） S1： 血氧值 校验错误或执行Fail返回：0xA4 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 例如：发送 0x09 01 01 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x09 66 00 00 00 19 01 00 00 05 00 00 00 25 00 00 00 00 00 00 00 00 5A 01 00 00 00 00 00 00 00 00 66 00 00 00：102步¦19 01 00 00：2.81KCAL¦05 00 00 00：0.05km¦5A ：90¦01 00：1.0℃¦00：血氧值，需搭配0x28指令使用才出值 |  | Command Format:0x09 AA BB 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Function: Start or stop the real-time step counting mode. |  |  |  |
|  |  |  |  |  | Parameters: |  |  |  |
|  |  |  |  |  | AA: |  |  |  |
|  |  |  |  |  | 1: Start real-time step counting. |  |  |  |
|  |  |  |  |  | 0: Stop real-time step counting. |  |  |  |
|  |  |  |  |  | BB: |  |  |  |
|  |  |  |  |  | 1: Enable real-time temperature. |  |  |  |
|  |  |  |  |  | 0: Disable real-time temperature. |  |  |  |
|  |  |  |  |  | Response: |  |  |  |
|  |  |  |  |  | Successful Response (valid checksum and success): |  |  |  |
|  |  |  |  |  | 0x09 AA BB CC DD EE FF GG HH II JJ KK LL 00 00 00 00 00 00 00 00 UU T1 T2 S1 00 |  |  |  |
| 11 |  | 实时计步/心率/血氧/体温数据传输模式 Real-time Step/HR/Oxygen/Temperature Data Transmission Mode |  |  | Failure Response (checksum error or failure in execution): 0xA4 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response Description: AA BB CC DD: Total step count. EE FF GG HH: Calories (display in KCAL, divide the value by 100 and retain two decimal places). II JJ KK LL: Walking distance (display in KM, divide the value by 100 and retain two decimal places). | JCV8 | 健康设置 Health Settings |  |
|  |  |  |  |  | UU: Heart rate value (big-endian format). |  |  |  |
|  |  |  |  |  | T1 T2: Temperature change value (precision of 0.1°C). |  |  |  |
|  |  |  |  |  | S1: Blood oxygen level (requires 0x28 command for data). |  |  |  |
|  |  |  |  |  | Example: |  |  |  |
|  |  |  |  |  | Command Sent: 0x09 01 01 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |
|  |  |  |  |  | Response Received: 0x09 66 00 00 00 19 01 00 00 05 00 00 00 25 00 00 00 00 00 00 00 00 5A 01 00 00 00 00 00 00 00 00 |  |  |  |
|  |  |  |  |  | 66 00 00 00: 102 steps |  |  |  |
|  |  |  |  |  | 19 01 00 00: 2.81 KCAL |  |  |  |
|  |  |  |  |  | 05 00 00 00: 0.05 km |  |  |  |
|  |  |  |  |  | 5A: 90 (Heart rate) |  |  |  |
|  |  |  |  |  | 01 00: 1.0°C (Temperature change) |  |  |  |
|  |  |  |  |  | 00: Blood oxygen value (requires pairing with 0x28 command for valid data) |  |  |  |
| 12 | 深 | 设置自动检测心率/血氧/HRV等的时间间隔 Set the time interval for automatic detection of heart rate, blood oxygen, HRV, etc. | 命令格式： 0x2A AA BB CC DD EE FF GG HH II 00 00 00 00 00 CRC 功能：设置运动时间段 。描述： AA：工作模式 0:关闭 2: 时间段内间隔工作方式 （自动测量间隔默认全天 ，睡眠模式时心率间隔自动变为 5分钟） BB：开始测量时间的小时部分 （24小时制）BCD码格式（如23点AA = 0x23） CC：开始测量时间的分钟部分 ，BCD码格式（如59分 CC = 0x59） DD：结束测量时间的小时部分 （24小时制）BCD码格式（如23点DD = 0x23） EE：结束测量时间的分钟部分 ，BCD码格式（如59分 EE = 0x59） FF：星期使能位:Bit0 = 0 表示星期天不使能 ，Bit0 = 1表示星期天使能 Bit1 = 0 表示星期一不使能，Bit1 = 1表示星期一使能 Bit2 = 0 表示星期二不使能，Bit2 = 1表示星期二使能 Bit3 = 0 表示星期三不使能，Bit3 = 1表示星期三使能 Bit4 = 0 表示星期四不使能，Bit4 = 1表示星期四使能 Bit5 = 0 表示星期五不使能，Bit5 = 1表示星期五使能 Bit6 = 0 表示星期六不使能，Bit6 = 1表示星期六使能 GG HH：当工作在工作模式 2时，多少时间测试一次，单位是分钟。每次测试时间是 1分钟。 II： 1:设置心率 2:设置血氧 4:HRV测量 命令回复： 校验正确且执行OK返回：0x2A 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 校验错误或执行Fail返回：0xAA 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 例如：发送 0x2A 02 00 00 23 59 7F 00 05 02 00 00 00 00 00 CRC 返回 0x2A 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  | Command format:0x2A AA BB CC DD EE FF GG HH II 00 00 00 00 00 CRC Function: Set exercise time period. Description: AA: Working mode 0: Off 2: Interval working mode (automatic measurement interval is set for the entire day, in sleep mode, heart rate interval is automatically set to 5 minutes) BB: Hour part of the start time for measurements (24-hour format, BCD code format, e.g., 23:00 → AA = 0x23) CC: Minute part of the start time for measurements, BCD code format (e.g., 59 minutes → CC = 0x59) DD: Hour part of the end time for measurements (24-hour format, BCD code format, e.g., 23:00 → DD = 0x23) EE: Minute part of the end time for measurements, BCD code format (e.g., 59 minutes → EE = 0x59) FF: Weekday enable bits: Bit0 = 0 means Sunday disabled, Bit0 = 1 means Sunday enabled Bit1 = 0 means Monday disabled, Bit1 = 1 means Monday enabled Bit2 = 0 means Tuesday disabled, Bit2 = 1 means Tuesday enabled Bit3 = 0 means Wednesday disabled, Bit3 = 1 means Wednesday enabled Bit4 = 0 means Thursday disabled, Bit4 = 1 means Thursday enabled Bit5 = 0 means Friday disabled, Bit5 = 1 means Friday enabled Bit6 = 0 means Saturday disabled, Bit6 = 1 means Saturday enabled GG HH: When working in mode 2, how often to test (in minutes). Each test takes 1 minute. II: Measurement type 1: Set heart rate 2: Set blood oxygen 4: HRV measurement Command Response: If checksum is correct and execution is OK, the response is: 0x2A 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC If checksum is incorrect or execution fails, the response is: 0xAA 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Example: Send: 0x2A 02 00 00 23 59 7F 00 05 02 00 00 00 00 00 CRC Response: 0x2A 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC | JCV8 | 健康设置 Health Settings |  |
|  |  | 圳市友宏科技有限公司版权 | 所有 ！保密文件，请严格遵守保密协议 ，请勿外传给第三方！ |  |  |  |  |  |

| 深圳市友宏科技有限公司 （保密） Shenzhen Youhong Technology Co., Ltd. (Confidential) . |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 蓝牙穿戴产品 API 协议 产品名称：Smart screenless wristband 型号：JCV8 Bluetooth Wearable Product API Protocol. Product Name:Smart screenless wristband. Model: JCV8 |  |  |  |  |  |  |
| 序号 Serial Number | 功能描述 Function Description | 命令格式描述 | Command format description | 适配产品类型 Compatible product types | 分类 Category | 备注 Remarks |
| 13 | 读取自动检测心率/血氧/HRV等的时间间隔 Get the time interval for automatic detection of heart rate, blood oxygen, HRV, etc. | 命令格式： 0x2B AA 00 00 00 00 00 00 00 00 00 00 00 00 CRC 功能：APP读取戒指所设置的时间段值 描述：AA :1、读取心率设置时间 2、读取设置血氧时间 4、读取设置HRV测量时间命令回复： 校验正确且执行OK返回：0x2B AA BB CC DD EE FF GG HH II 00 00 00 00 CRC 回复描述：（返回数据为大端） AA：工作模式，0：关闭 2： 时间段内间隔工作方式 。 BB：为开始测量时间的小时部分 （24小时制）。BCD码格式（如23点AA = 0x23） CC：为开始测量时间的分钟部分 ，BCD码格式（如59分 BB = 0x59） DD：为结束测量时间的小时部分 （24小时制）。BCD码格式（如23点AA = 0x23） EE：为结束测量时间的分钟部分 ，BCD码格式（如59分 BB = 0x59） FF：为星期使能位：Bit0 = 0 表示星期天不使能，Bit0 = 1表示星期天使能 Bit1 = 0 表示星期一不使能 ，Bit1 = 1表示星期一使能 Bit2 = 0 表示星期二不使能 ，Bit2 = 1表示星期二使能 Bit3 = 0 表示星期三不使能 ，Bit3 = 1表示星期三使能 Bit4 = 0 表示星期四不使能 ，Bit4 = 1表示星期四使能 Bit5 = 0 表示星期五不使能 ，Bit5 = 1表示星期五使能 Bit6 = 0 表示星期六不使能 ，Bit6 = 1表示星期六使能 GG HH：当工作在工作模式 2时，多少时间测试一次 ，单位是分钟，每次测试时间是 1分钟 II： 1:设置心率 2:设置血氧 4:HRV 测量 校验错误或执行Fail返回：0xAB 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 例如：发送 0x2B 02 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x2B 02 00 00 23 59 7F 00 05 02 00 00 00 00 CRC 02：时间段内间隔工作方式 ¦00 00：开始测量时间 00:00¦23 59：结束测量时间 23:59¦7F：0111 1111 ，全天工作 ¦00 05：隔5分钟自动检测一次¦02 ：血氧测量模式 | Command format:0x2B AA 00 00 00 00 00 00 00 00 00 00 00 00 CRC Function: The APP reads the time period settings from the ring. Description: AA: 1: Read heart rate setting time 2: Read blood oxygen setting time 4: Read HRV measurement setting time Command Response: If checksum is correct and execution is OK, the response is: 0x2B AA BB CC DD EE FF GG HH II 00 00 00 00 CRC Response description (data is returned in big-endian format): AA: Working mode 0: Off 2: Interval working mode (within the specified time period) BB: Hour part of the start time for measurements (24-hour format), BCD code format (e.g., 23:00 → AA = 0x23) CC: Minute part of the start time for measurements, BCD code format (e.g., 59 minutes → BB = 0x59) DD: Hour part of the end time for measurements (24-hour format), BCD code format (e.g., 23:00 → AA = 0x23) EE: Minute part of the end time for measurements, BCD code format (e.g., 59 minutes → BB = 0x59) FF: Weekday enable bits: Bit0 = 0 means Sunday disabled, Bit0 = 1 means Sunday enabled Bit1 = 0 means Monday disabled, Bit1 = 1 means Monday enabled Bit2 = 0 means Tuesday disabled, Bit2 = 1 means Tuesday enabled Bit3 = 0 means Wednesday disabled, Bit3 = 1 means Wednesday enabled Bit4 = 0 means Thursday disabled, Bit4 = 1 means Thursday enabled Bit5 = 0 means Friday disabled, Bit5 = 1 means Friday enabled Bit6 = 0 means Saturday disabled, Bit6 = 1 means Saturday enabled GG HH: When working in mode 2, how often to test (in minutes). Each test takes 1 minute. II: Measurement type 1: Set heart rate 2: Set blood oxygen 4: HRV measurement Command Error or Fail Response: If checksum is incorrect or execution fails, the response is: 0xAB 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Example: Send: 0x2B 02 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0x2B 02 00 00 23 59 7F 00 05 02 00 00 00 00 CRC Explanation: 02: Interval working mode within the time period 00 00: Start measurement time 00:00 23 59: End measurement time 23:59 7F: 0111 1111 (All days enabled) 00 05: Automatic test every 5 minutes | JCV8 | 健康设置 Health Settings |  |
| 14 | 获得计步总数据 Get total step count data | 命令格式： 0x51 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC AA： 00:读取记步总数据 功能：获得全部总数据 （获取的数据为小端 ） 返回数据：（可变长指令） 0x51 ID YY MM DD S1 S2 S3 S4 T1 T2 T3 T4 D1 D2 D3 D4 K1 K2 K3 K4 00 00 00 00 00 00 长度不固定，最小单位是27个字节，无CRC校验（如只有一条数据 ，则返回29个字节，以51 FF为结尾）步数数据包最多 （数据字节为小端） ID：编号，倒数第X天的总数据 0表示当天，是实时数据。范围是0到15，共15天的数据（数据存储的方式为最 大15天，如第16天将覆盖第一天的数据 ，以此类推） YY MM DD：表示数据的日期:年月日 S1 S2 S3 S4：步数 T1 T2 T3 T4：运动时间，单位是秒（累积运动时长） D1 D2 D3 D4：距离，单位是0.01KM K1 K2 K3 K4：卡路里，单位是0.01KCAL 例如：发送 0x51 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x51 00 24 08 27 2E 00 00 00 11 00 00 00 03 00 00 00 93 00 00 00 00 00 00 00 00 00 51 FF 00：ID为00 （如有长数据以51 ID开头为一条数据进行分隔 ）¦24 08 27：2024年8月27日¦2E 00 00 00：226步¦11 00 00 00：17秒（运动时长）¦03 00 00 00： 0.48km¦93 00 00 00:1.47KCAL | Command format:0x51 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC AA: 00: Read total step count data Function: Retrieve all total data (the data returned is in little-endian format). Response Data Format: (Variable length instruction) 0x51 ID YY MM DD S1 S2 S3 S4 T1 T2 T3 T4 D1 D2 D3 D4 K1 K2 K3 K4 00 00 00 00 00 00 The length is variable, with the minimum being 27 bytes. There is no CRC check. If only one data entry is returned, the response is 29 bytes, ending with 0x51 FF. The maximum length for step count data is (data bytes in little-endian format). Response Description: ID: Identifier, representing the total data from the X-th previous day. 0 means the current day (real-time data). The range is from 0 to 15, representing 15 days of data (the storage system keeps up to 15 days, and the 16th day will overwrite the 1st day, and so on). YY MM DD: Represents the date of the data: Year, Month, Day. S1 S2 S3 S4: Step count data. T1 T2 T3 T4: Exercise time (in seconds, accumulated exercise duration). D1 D2 D3 D4: Distance (in 0.01 km). K1 K2 K3 K4: Calories burned (in 0.01 kcal). Example: Send: 0x51 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0x51 00 24 08 27 2E 00 00 00 11 00 00 00 03 00 00 00 93 00 00 00 00 00 00 00 00 00 51 FF Explanation: 00: ID is 00 (if there’s more data, it will be separated with 0x51 ID). 24 08 27: Date is August 27, 2024. 2E 00 00 00: 226 steps. 11 00 00 00: 17 seconds of exercise time. 03 00 00 00: 0.48 km. 93 00 00 00: 1.47 kcal burned. | JCV8 | 健康设置 Health Settings |  |
| 15 | 获得步数详细数据（一条数据包含十分钟内每分钟的具体步数 ） Obtain detailed step count data (one entry contains the specific step count for each minute within a 10-minute period) | 命令格式： 0x52 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA： 99:删除步数详细数据 00:读最近的步数详细数据 （读最近时间戳开始的 50条数据） YY MM DD HH mm SS:数据时间戳，发送00时携带时间戳可以只查询指定时间戳后的新数据 （第一次同步需要返回全部数据 ，00后的时间戳为历史数据中所存在的时间戳） 02:继续上次读的位置下一段数据 （读上一次读到的 50条数据的下一段 50条数据）功能：获得步数详细数据 返回数据：（可变长指令）0x52 ID1 ID2 YY MM DD HH mm SS S1 S2 K1 K2 D1 D2 SD1 SD2 SD3 ... SD10（无CRC校验） 最小单位是25个字节。一个步数详细数据蓝牙数据包最多包含 9条数据，如数据包长度 < 发送数据总数，则按照顺序每 9条数据一个数据包 ，如一共19条数据，第0条数据至第8条数据为第一个数据包 ，第9条数据至第 17条数据为第二个数据包 ，第18条数据后面再加一条 52 FF指令为第三个数据包 。52 FF表示数据结束，下述蓝牙数据包分包方案与此规则相似 开始计步时，每10分钟会存储一次数据 ，每分钟会记录步数数据 ，如果没有步数，不计数（返回的测量数据为小端 ） ID1 ID2： 读数据编号 YY MM DD HH mm SS：表示的日期：年月日时分秒 S1 S2： 步数 K1 K2： 卡路里，单位是0.01KCAL D1 D2： 距离，单位是0.01KM SD1 SD2 SD3 ... SD10 ：表示这10分钟内每一分钟的步数 例如：发送 0x52 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x52 00 00 24 08 08 23 58 19 1D 00 25 00 00 00 0A 13 00 00 00 00 00 00 00 00 52 02 00 24 08 08 23 43 19 0D 00 19 00 01 00 0D 00 00 00 00 00 00 00 00 00 52 03 00 24 08 08 22 46 19 32 00 B2 00 03 00 16 04 00 00 00 0A 00 0E 00 00 52 04 00 24 08 08 22 33 19 0E 00 2B 00 00 00 0E 00 00 00 00 00 00 00 00 00 52 05 00 24 08 08 20 22 19 28 00 62 00 01 00 28 00 00 00 00 00 00 00 00 00 52 06 00 24 08 08 20 08 19 0C 00 2D 00 01 00 0A 02 00 00 00 00 00 00 00 00 52 07 00 24 08 27 11 37 19 1D 00 64 00 02 00 0D 00 10 00 00 00 00 00 00 00 52 08 00 24 08 27 09 21 19 1C 00 6A 00 03 00 0D 0F 00 00 00 00 00 00 00 00 52 09 00 24 08 27 09 10 19 12 00 28 00 00 00 0E 04 00 00 00 00 00 00 00 00 52 FF 将返回数据以52 XX XX进行分段 09 00：数据ID¦24 08 27 09 10 19：24年8月27日9点10分19秒¦12 00：33步¦28 00：1.30KCAL¦ 00 00：0km¦0E 04···00：0E表示第一分钟的步数 ，04表示第二分钟的步数 ，以此类推，每一小段数据为 10分钟的数据 | Command format:0x52 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA: 99: Delete detailed step count data 00: Read the most recent detailed step count data (retrieve the last 50 data entries based on the timestamp) YY MM DD HH mm SS: Data timestamp. Sending "00" with a timestamp allows you to query only data after a specified timestamp (for the first sync, return all data, and the timestamp after "00" will be from the historical data). 02: Continue reading from the last read position (retrieve the next 50 entries after the last batch of 50 entries). Function: Retrieve detailed step count data. Response Data Format: (Variable-length instruction) 0x52 ID1 ID2 YY MM DD HH mm SS S1 S2 K1 K2 D1 D2 SD1 SD2 SD3 ... SD10 (No CRC check) The minimum length is 25 bytes. A single detailed step count Bluetooth data packet can contain up to 9 data entries. If the packet length is less than the total data being sent, it will be divided into separate packets, with each containing up to 9 entries. For example, if there are 19 entries, the first packet will contain entries 0-8, the second packet will contain entries 9-17, and the third packet will contain entry 18 followed by a 0x52 FF instruction to indicate the end of the data. The packet division scheme follows a similar rule. When step counting begins, data is stored every 10 minutes, and the step count for each minute is recorded. If no steps are taken in a minute, it is not counted. (The returned measurement data is in little-endian format.) Response Data Description: ID1 ID2: Data read identifier YY MM DD HH mm SS: Date and time of the data: Year, Month, Day, Hour, Minute, Second. S1 S2: Step count for the 10-minute period. K1 K2: Calories burned (in 0.01 kcal). D1 D2: Distance traveled (in 0.01 km). SD1 SD2 ... SD10: Step count for each minute in the 10-minute period. Example: Send: 0x52 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0x52 00 00 24 08 08 23 58 19 1D 00 25 00 00 00 0A 13 00 00 00 00 00 00 00 00 0x52 02 00 24 08 08 23 43 19 0D 00 19 00 01 00 0D 00 00 00 00 00 00 00 00 00 ··· 0x52 08 00 24 08 27 09 21 19 1C 00 6A 00 03 00 0D 0F 00 00 00 00 00 00 00 00 0x52 09 00 24 08 27 09 10 19 12 00 28 00 00 00 0E 04 00 00 00 00 00 00 00 00 0x52 FF Explanation: 09 00: Data ID. 24 08 27 09 10 19: Date and time is August 27, 2024, at 9:10:19. 12 00: Step count is 33. 28 00: 1.30 kcal burned. 00 00: 0 km traveled. 0E 04 ... 00: Step counts for each minute within the 10-minute period. Each data packet contains up to 9 data entries. | JCV8 | 健康设置 Health Settings |  |
| 16 | 获得睡眠历史数据 Obtain historical sleep data | 命令格式： 0x53 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA： 99:删除睡眠详细数据 00:读最近的睡眠详细数据 （读最近时间戳开始的 50条数据） YY MM DD HH mm SS:数据时间戳，发送00时携带时间戳可以只查询指定时间戳后的新数据 （第一次同步需要返回全部数据 ，00后的时间戳为历史数据中所存在的时间戳） 02:继续上次读的位置下一段数据 （读上一次读到的 50条数据的下一段 50条数据） 返回数据：（可变长指令） 0x53 ID1 ID2 YY MM DD HH mm SS LEN SD1 SD2 SD3 ... SD120 （无CRC校验）最小单位是130 个字节，一个数据包为一条数据 ，返回0x54 FF表示数据结束 ID1 ID2： 读数据编号 YY MM DD HH mm SS：表示数据的日期 ：年月日时分秒 LEN： 长度表示睡眠数据的长度 （实际数据长度只取LEN数量长度，LEN为有效数据长度 ，16进制） SD1 SD2 SD3 ... SD120：每一个数字表示 1分钟的睡眠质量，最多表示2小时的睡眠数据（01=深睡，02=浅睡，03=眼动，有效数据内的其余数字表示清醒 ，例00、05 。如果数据中断，app端进行数据补偿） 例如：发送 0x53 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x53 00 00 24 08 23 13 36 00 36 05 05 05 05 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 05 02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00：数据编号¦24 08 23 13 36 00：数据的起始时间为 24年8月23日13时36分0秒¦36：36个字节长度的有效数据 （含义为取LE后面36个字节为有效数据，其余数据为无效数据）¦05 ··· 02：每分钟的睡眠质量（01=深睡，02=浅睡，03=眼动，其余数据表示清醒） | Command Format:0x53 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA: 99: Delete detailed sleep data 00: Read the most recent sleep detailed data (retrieve the last 50 data entries starting from the specified timestamp) YY MM DD HH mm SS: Data timestamp. Sending "00" with a timestamp allows you to query only data after a specified timestamp (for the first sync, return all data, and the timestamp after "00" will be from the historical data). 02: Continue reading from the last read position (retrieve the next 50 entries after the last batch of 50 entries). Function: Retrieve detailed sleep data. Response Data Format: (Variable-length instruction) 0x53 ID1 ID2 YY MM DD HH mm SS LEN SD1 SD2 SD3 ... SD120 (No CRC check) The minimum length is 130 bytes. Each data packet represents one entry, and a response ending with 0x54 FF signifies the end of the data. ID1 ID2: Data read identifier YY MM DD HH mm SS: Date and time of the data: Year, Month, Day, Hour, Minute, Second. LEN: Length that indicates the valid length of sleep data (only the first LEN bytes are valid data, in hexadecimal). SD1 SD2 SD3 ... SD120: Each number represents the sleep quality for one minute. Up to 2 hours (120 minutes) of sleep data can be recorded: 01 = Deep sleep 02 = Light sleep 03 = REM Other numbers (e.g., 00, 05) represent awake states. If data is interrupted, the app will compensate for the missing data. Example: Send: 0x53 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0x53 00 00 24 08 23 13 36 00 36 05 05 05 05 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 02 05 02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 0x53 FF Explanation: 00 00: Data ID 24 08 23 13 36 00: The start time of the data is August 23, 2024, at 13:36:00. 36: The valid data length is 36 bytes. 05 ... 02: Sleep quality for each minute (01 = Deep sleep, 02 = Light sleep, 03 = REM sleep, and other values represent awake). | JCV8 | 健康设置 Health Settings |  |
| 17 | 获得心率数据（读取详细的心率数 据，长时间测量的心率详细数据 ） Obtain heart rate data (retrieve detailed heart rate data, including long-term measurement heart rate details) | 命令格式： 0x54 AA 00 00 YY MM DD HH mm SS 00 00 00 00 TT CRC AA： 99:删除心率详细数据 00:读最近的心率详细数据 （读最近时间戳开始的 50条数据） YY MM DD HH mm SS:数据时间戳，发送00时携带时间戳可以只查询指定时间戳后的新数据 （第一次同步需要返回全部数据 ，00后的时间戳为历史数据中所存在的时间戳） 02:继续上次读的位置下一段数据 （读上一次读到的 50条数据的下一段 50条数据） TT: 一次最多传输包数控制 ，50的整数倍数，如果没有，默认是50包。如1：50条，2，100条，3，150条...返回数据：（可变长指令） 0x54 ID1 ID2 YY MM DD HH mm SS SD1 SD2 SD3 ... SD15（无CRC校验） 最小单位是21 个字节。一个步数详细数据蓝牙数据包最多包含 10条数据末尾54 FF表示数据结束或返回 0x54 FF表示数据结束 ID1 ID2： 读数据编号 YY MM DD HH mm SS： 表示数据的日期：年月日时分秒 SD1 SD2 SD3 ... SD15：每5s保存一个心率值 ，总共15个心率值，表示75秒的心率数据（16进制）例如：发送 0x54 00 00 00 00 00 00 00 00 00 00 00 00 00 00 54 返回 0x54 00 00 24 08 08 22 59 27 4C 4E 00 00 00 00 00 00 00 00 00 00 00 00 00 54 01 00 24 08 08 22 58 12 4C 49 4A 4D 50 51 50 4F 4E 4F 4C 4B 4B 4C 4C 54 FF 00 00：数据编号¦24 08 08 22 59 27：时间，24年8月8日22时59分27秒¦4C···00：十六进制换算 4C=76 每个字节表示每 5s保存的心率值 | Command Format:0x54 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA: 99: Delete detailed heart rate data 00: Read the most recent heart rate detailed data (retrieve the last 50 data entries starting from the specified timestamp) YY MM DD HH mm SS: Data timestamp. Sending "00" with a timestamp allows you to query only data after a specified timestamp (for the first sync, return all data, and the timestamp after "00" will be from the historical data). 02: Continue reading from the last read position (retrieve the next 50 entries after the last batch of 50 entries). Function: Retrieve detailed heart rate data. Response Data Format: (Variable-length instruction) 0x54 ID1 ID2 YY MM DD HH mm SS SD1 SD2 SD3 ... SD15 (No CRC check) The minimum length is 21 bytes. Each data packet represents one entry, and a response ending with 0x54 FF signifies the end of the data. ID1 ID2: Data read identifier YY MM DD HH mm SS: Date and time of the data: Year, Month, Day, Hour, Minute, Second. SD1 SD2 SD3 ... SD15: Each value represents the heart rate recorded every 5 seconds. There are 15 heart rate values in total, which cover 75 seconds of heart rate data. The data is in hexadecimal. Example: Send: 0x54 00 00 00 00 00 00 00 00 00 00 00 00 00 00 54 Response: 0x54 00 00 24 08 08 22 59 27 4C 4E 00 00 00 00 00 00 00 00 00 00 00 00 00 0x54 01 00 24 08 08 22 58 12 4C 49 4A 4D 50 51 50 4F 4E 4F 4C 4B 4B 4C 4C 0x54 FF Explanation: 00 00: Data ID 24 08 08 22 59 27: The time of the data is August 8, 2024, at 22:59:27. 4C...00: The heart rate data is stored in hexadecimal format. For example, 4C in hexadecimal equals 76 in decimal (heart rate value). Each byte represents a heart rate recorded every 5 seconds | JCV8 | 健康设置 Health Settings |  |
| 18 | 获得心率测量历史数据 （测量的单个心率值） Obtain heart rate measurement history data (individual heart rate values measured) | 命令格式： 0x55 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA： 99:删除心率数据 00:读最近的心率数据 （读最近时间戳开始的 50条数据） YY MM DD HH mm SS:数据时间戳，发送00时携带时间戳可以只查询指定时间戳后的新数据 （第一次同步需要返回全部数据 ，00后的时间戳为历史数据中所存在的时间戳） 02:继续上次读的位置下一段数据 （读上一次读到的 50条数据的下一段 50条数据） 返回数据：（可变长指令） 0x55 ID1 ID2 YY MM DD HH mm SS HH（无CRC校验）（返回数据以55开头分段）最小数据是10个字节（即最少返回 12个字节，55 FF结尾表示结束） ID1 ID2： 读数据编号 YY MM DD HH mm SS：表示数据的日期 ：年月日时分秒 HH： 心率值（16进制） 例如：发送 0x55 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x55 00 00 24 08 27 09 01 30 46 0x55 FF 00 00：数据编号¦24 08 27 09 01 30：时间24年8月27日9时1分30秒¦46：心率值70（16进制换算） | Command Format:0x55 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA: 99: Delete heart rate data 00: Read the most recent heart rate data (retrieve the last 50 data entries starting from the specified timestamp) YY MM DD HH mm SS: Data timestamp. Sending "00" with a timestamp allows you to query only data after a specified timestamp (for the first sync, return all data, and the timestamp after "00" will be from the historical data). 02: Continue reading from the last read position (retrieve the next 50 entries after the last batch of 50 entries). Function: Retrieve heart rate data. Response Data Format: (Variable-length instruction) 0x55 ID1 ID2 YY MM DD HH mm SS HH (No CRC check) The minimum length is 10 bytes. Each data packet represents one entry, and a response ending with 0x55 FF signifies the end of the data. ID1 ID2: Data read identifier YY MM DD HH mm SS: Date and time of the data: Year, Month, Day, Hour, Minute, Second. HH: Heart rate value in hexadecimal. Example: Send: 0x55 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0x55 00 00 24 08 27 09 01 30 46 0x55 FF Explanation: 00 00: Data ID 24 08 27 09 01 30: The time of the data is August 27, 2024, at 09:01:30. 46: The heart rate value 70 (in hexadecimal 46). | JCV8 | 健康设置 Health Settings |  |

| 深圳市友宏科技有限公司 （保密） Shenzhen Youhong Technology Co., Ltd. (Confidential) . |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 蓝牙穿戴产品 API 协议 产品名称：Smart screenless wristband 型号：JCV8 Bluetooth Wearable Product API Protocol. Product Name:Smart screenless wristband. Model: JCV8 |  |  |  |  |  |  |
| 序号 Serial Number | 功能描述 Function Description | 命令格式描述 | Command format description | 适配产品类型 Compatible product types | 分类 Category | 备注 Remarks |
| 19 | 获得HRV测量历史数据 Obtain HRV measurement history data | 命令格式： 0x56 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA： 99:删除HRV数据 00:读最近的HRV数据（读最近时间戳开始的 50条数据） YY MM DD HH mm SS:数据时间戳，发送00时携带时间戳可以只查询指定时间戳后的新数据 （第一次同步需要返回全部数据 ，00后的时间戳为历史数据中所存在的时间戳） 02:继续上次读的位置下一段数据 （读上一次读到的 50条数据的下一段 50条数据） 返回数据：（可变长指令） 0x56 ID1 ID2 YY MM DD HH mm SS D1 00 D3 D4 P1 P2（无CRC校验）最小单位是15 个字节。一个步数详细数据蓝牙数据包最多包含 16条数据。 ID1 ID2： 读数据编号 YY MM DD HH mm SS：表示数据的日期 ：年月日时分秒 D1： HRV 值（16进制） D3： 心率值（16进制） D4： 疲劳度（16进制） P1： 血压高压 （血压不保证精度） P2： 血压低压 （血压不保证精度） 例如：发送 0x56 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x56 00 00 24 08 09 00 59 30 40 00 4D 1E 75 3E 56 01 00 24 08 08 22 59 30 32 00 4E 38 76 3F 56 02 00 24 08 27 08 59 30 33 00 00 50 00 00 56 FF 02 00：数据编号¦24 08 27 08 59 30： 时间24年8月27日8点59分30秒¦33：HRV值51¦00：心率值¦50：疲劳值 80 ¦76 3F：血压高压118，血压低压 63 以56 FF结尾或者回复 0x56 FF结束 | Command Format:0x56 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA: 99: Delete HRV data 00: Read the most recent HRV data (retrieve the last 50 data entries starting from the specified timestamp) YY MM DD HH mm SS: Data timestamp. Sending "00" with a timestamp allows you to query only data after a specified timestamp (for the first sync, return all data, and the timestamp after "00" will be from the historical data). 02: Continue reading from the last read position (retrieve the next 50 entries after the last batch of 50 entries). Function: Retrieve HRV data. Response Data Format: (Variable-length instruction) 0x56 ID1 ID2 YY MM DD HH mm SS D1 00 D3 D4 P1 P2 (No CRC check) The minimum length is 15 bytes. Each data packet represents one entry, and a response ending with 0x56 FF signifies the end of the data. ID1 ID2: Data read identifier YY MM DD HH mm SS: Date and time of the data: Year, Month, Day, Hour, Minute, Second. D1: HRV value (in hexadecimal) D3: Heart rate value (in hexadecimal) D4: Fatigue level (in hexadecimal) P1: Systolic blood pressure (Note: Blood pressure values may not be precise) P2: Diastolic blood pressure (Note: Blood pressure values may not be precise) Example: Send: 0x56 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0x56 00 00 24 08 09 00 59 30 40 00 4D 1E 75 3E 0x56 01 00 24 08 08 22 59 30 32 00 4E 38 76 3F 0x56 02 00 24 08 27 08 59 30 33 00 00 50 00 00 0x56 FF Explanation: 00 00: Data ID 24 08 27 08 59 30: The data timestamp is August 27, 2024, at 08:59:30. 33: HRV value 51 (in hexadecimal 33) 00: Heart rate value 0 (in hexadecimal 00) 50: Fatigue level 80 (in hexadecimal 50) 76 3F: Blood pressure: systolic 118, diastolic 63 (in hexadecimal 76 3F) | JCV8 | 健康设置 Health Settings |  |
| 20 | 获得日常温度历史数据 Obtain daily temperature history data | 命令格式： 0x62 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA： 99:删除温度详细数据 00:读最近的温度数据 （读最近时间戳开始的 50条数据） YY MM DD HH mm SS:数据时间戳，发送00 时携带时间戳可以只查询指定时间戳后的新数据 （第一次同步需要返回全部数据 ，00后的时间戳为历史数据中所存在的时间戳） 02:继续上次读的位置下一段数据 （读上一次读到的 50条数据的下一段 50条数据）返回数据：（可变长指令） 0JCV82 ID1 ID2 YY MM DD HH mm SS T1 T2 最小单位是11个字节。最多每个蓝牙数据包包含 16条数据 ID1 ID2：数据编号 YY MM DD HH mm SS：时间:年月日时分秒 T1 T2 ：温度值 ℃，小端，保留1位小数 例如：发送 0JCV82 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0JCV82 00 00 24 08 09 00 59 59 5A 01 49 01 59 01 62 FF 0A 00：数据编号¦24 08 27 08 56 59：时间24年8月27日8点56分59秒¦5A 01 49 01 59 01：分别对应三个NTC温度34.6℃返回0JCV82 FF或数据末尾为 62 FF数据结束 | Command Format:0x62 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA: 99: Delete temperature detailed data 00: Read the most recent temperature data (retrieve the last 50 entries starting from the specified timestamp) YY MM DD HH mm SS: Data timestamp. Sending "00" with a timestamp allows you to query only data after a specified timestamp (for the first sync, return all data, and the timestamp after "00" will be from the historical data). 02: Continue reading from the last read position (retrieve the next 50 entries after the last batch of 50 entries). Function: Retrieve detailed temperature data. Response Data Format: (Variable-length instruction) 0JCV82 ID1 ID2 YY MM DD HH mm SS T1 T2 T3 T4 T5 T6 (No CRC check) The minimum length is 15 bytes. Each data packet represents 16 entry, and a response ending with 0JCV82 FF signifies the end of the data. ID1 ID2: Data read identifier YY MM DD HH mm SS: Date and time of the data: Year, Month, Day, Hour, Minute, Second. T1 T2 T3 T4 T5 T6: Temperature value in Celsius (°C), stored as a 2-byte value in little-endian format, with one decimal place of precision. For example, 59 01 represents 34.5°C. Example: Send: 0JCV82 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0JCV82 00 00 24 08 09 00 59 59 5A 01 49 01 59 01 ~~~ 0JCV82 0A 00 24 08 27 08 56 59 D9 00 E0 00 D8 00 0JCV82 FF Explanation: 00 00: Data ID 24 08 27 08 56 59: Timestamp for the data is August 27, 2024, at 08:56:59. 5A 01: Temperature value of 34.6°C (little-endian representation) 49 01: Temperature value of 32.9°C (little-endian representation) 59 01: Temperature value of 34.5°C (little-endian representation). The data packets continue until the last packet with 0JCV82 FF, which signifies the end of the data | JCV8 | 健康设置 Health Settings |  |
| 21 | 获得血氧历史数据（自动测试数据） Obtain historical blood oxygen data (automatic test data). | 命令格式： 0x66 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA： 99:删除血氧数据 00:读最近的血氧数据 （读最近时间戳开始的 50条数据） YY MM DD HH mm SS:数据时间戳，发送00 时携带时间戳可以只查询指定时间戳后的新数据 （第一次同步需要返回全部数据 ，00后的时间戳为历史数据中所存在的时间戳） 02:继续上次读的位置下一段数据 （读上一次读到的 50条数据的下一段 50条数据） 返回数据：（可变长指令） 0JCV86 ID1 ID2 YY MM DD HH mm SS HH（以66-HH为一段数据进行分段 ）最小单位是10个字节。最多每个蓝牙数据包包含 22条数据 ID1 ID2： 读数据编号，小端 YY MM DD HH mm SS：表示数据的日期 ：年月日时分秒 HH： 血氧值（16进制） 例如：发送 0JCV86 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 66 00 00 24 08 09 00 00 23 61 66 01 00 24 08 27 09 00 19 62 66 FF 01 00：数据编号¦24 08 27 09 00 19：时间，24年8月27日9时0分19秒¦62：血氧值98（浓度,16进制）返回0JCV86 FF或以66 FF结尾为数据结束 | Command Format:0x66 AA 00 00 YY MM DD HH mm SS 00 00 00 00 00 CRC AA: 99: Delete blood oxygen data 00: Read the most recent blood oxygen data (retrieve the last 50 entries starting from the specified timestamp) YY MM DD HH mm SS: Data timestamp. Sending "00" with a timestamp allows you to query only data after a specified timestamp (for the first sync, return all data, and the timestamp after "00" will be from the historical data). 02: Continue reading from the last read position (retrieve the next 50 entries after the last batch of 50 entries). Response Data Format: (Variable-length instruction) 0JCV86 ID1 ID2 YY MM DD HH mm SS HH (Data is segmented with 0JCV86 -HH for each segment) The minimum length is 10 bytes. Each Bluetooth data packet can contain up to 22 data entries, with each entry representing one data point. ID1 ID2: Data read identifier (little-endian format) YY MM DD HH mm SS: Date and time of the data: Year, Month, Day, Hour, Minute, Second HH: Blood oxygen value (in hexadecimal). This is the blood oxygen saturation level. Example: Send: 0JCV86 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0JCV86 00 00 24 08 09 00 00 23 61 0JCV86 01 00 24 08 27 09 00 19 62 0JCV86 FF Explanation: 00 00: Data ID 24 08 27 09 00 19: Timestamp for the data is August 27, 2024, at 09:00:19. 62: Blood oxygen value of 98% (in hexadecimal, 0JCV82 equals 98). The data packets continue until the last packet with 0JCV86 FF, which signifies the end of the data | JCV8 | 健康设置 Health Settings |  |
| 22 | 多参数综合健康测量 （心率/血氧 /HRV/ 血压/压力等） Multi-parameter comprehensive health measurement (heart rate, blood oxygen, HRV, blood pressure, stress, etc.) | 命令格式： 0x28 AA BB 00 CC DD EE 00 00 00 00 00 00 00 00 CRC 功能：APP向戒指发起心率、血氧、血压测量 AA: 0x01,HRV测量 BB: 1开启测量 0 关闭测量 CC DD：测试时长，单位秒，高位在后，当<30秒时，戒指使用30秒时长。 0x02，心率测量 BB: 1开启测量 0 关闭测量 CC DD：测试时长，单位秒，高位在后，当<30秒时，戒指使用30秒时长。 0x03，（X3A:反射血氧测量 ,X3H: 透射血氧测量） BB: 1开启测量 0 关闭测量 CC DD：测试时长，单位秒，高位在后，当<30秒时，戒指使用30秒时长。 EE: 1: 表示连续测试SPO2（同时需要CC DD时间超过5分钟）.需要保存数据，30S保存一个， 0x04，ECG测试 BB: 1开启测量 0 关闭测量 CC DD：测试时长，单位秒，高位在后，当<30秒时，戒指使用30秒时长。 EE: 1: 表示连续测试SPO2（同时需要CC DD时间超过5分钟）.需要保存数据，30S保存一个， 0x80，状态查询 返回28 80 AA BB 00 00 00 00 00 00 CC DD EE… AA,测试模式 1：心率 2： HRV，3，SPO2 CC: 连续测试SPO2状态，0，关闭，1 （28 03开的连续SPO2.） 2：（28 04开的连接SPO2） DD EE: 连续测试SPO2剩余工作时间，单位是秒，高位在后 |  | JCV8 | 健康设置 Health Settings |  |
| 23 | 运动模式控制开关 Exercise mode control switch | 命令格式： 0x19 AA BB CC DD 00 00 00 00 00 00 00 00 00 00 CRC AA: 1开始 2：暂停 3：继续 4：结束 5：查询状态 BB： 运动模式类型； - Run - Cycling - Badminton - Football - Tennis - Yoga - Meditation - Dance - Basketball - Walk - Workout - Cricket - Hiking - Aerobics - Ping-Pong ··· 如果BB = 6，即冥想模式 CC： 级别选择，可选范围：0、1、2（前提：BB=6 ，冥想模式） DD： 时间段选择，单位分钟其他模式CC DD置0 返回数据：0x19 AA YY MM DD HH mm SS 00 00 00 00 00 00 00 CRC AA: 1: 成功 0：失败,如果戒指在测试血压或已经在运动模式时通过 APP重新开始运动会返回失败 。 当 APP发送指令AA=4或者AA=5 为查询状态时 返回数据：0x19 AA BB YY MM DD HH mm SS 00 00 00 00 00 00 AA: 运动状态 BB：运动模式 YY MM DD HH mm SS：开始运动的时间点 校验错误返回：0xA6 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 例如：发送 0x19 01 00 00 00 00 00 00 00 00 00 00 00 00 00 1A 返回 0x19 01 24 09 03 15 51 47 00 00 00 00 00 00 00 F7 01：表示成功打开所设置的运动类型的运动模式 ¦24 09 03 15 51 47：表示24年9月3日15时51分47秒随后每秒返回一个 18开头的数据 例2： 发送 0x19 04 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x19 01 00 24 09 03 15 51 47 00 00 00 00 00 00 F7 01：成功关闭模式¦00：跑步模式¦24 09 03 15 51 47：24年9月3日15时51分47秒 | Command Format :0x19 AA BB CC DD 00 00 00 00 00 00 00 00 00 00 CRC AA (Action) 1: Start the exercise mode 2: Pause the exercise mode 3: Resume the exercise mode 4: End the exercise mode 5: Query the current exercise mode status BB (Exercise Mode Type): 0: Run 1: Cycling 2: Badminton 3: Football 4: Tennis 5: Yoga 6: Meditation (for meditation mode, see specific parameters for CC and DD) 7: Dance 8: Basketball 9: Walk 10: Workout 11: Cricket 12: Hiking 13: Aerobics 14: Ping-Pong (Additional modes can be added) CC (Level, optional for Meditation Mode): 0, 1, 2: Meditation levels (used only when BB = 6 for meditation mode) DD (Duration in minutes, optional for Meditation Mode): Time in minutes (used only when BB = 6 for meditation mode) For other exercise modes (non-meditation modes), CC and DD are set to 0. Response Format: 0x19 AA YY MM DD HH mm SS 00 00 00 00 00 00 00 CRC AA: 1: Success (operation successful) 0: Failure (e.g., if there is a conflict, like trying to start a new mode while one is already running, or if the device is in another mode like blood pressure testing) YY MM DD HH mm SS: Timestamp of when the exercise mode was started. Examples: Start Exercise Mode (e.g., Running Mode): Command:0x19 01 00 00 00 00 00 00 00 00 00 00 00 00 00 1A AA = 1: Start mode BB = 00: Running mode (Run) CC and DD = 0: No additional parameters Response:0x19 01 24 09 03 15 51 47 00 00 00 00 00 00 00 F7 AA = 01: Success 24 09 03 15 51 47: Timestamp (24th September 2023, 15:51:47) Pause the Exercise Mode: Command:0x19 02 00 00 00 00 00 00 00 00 00 00 00 00 00 2B Response:0x19 01 00 24 09 03 15 51 47 00 00 00 00 00 00 F7 AA = 01: Success (mode paused) BB = 00: Mode type (e.g., Run) Timestamp: 24th September 2023, 15:51:47 Query the Current Mode Status: Command:0x19 05 00 00 00 00 00 00 00 00 00 00 00 00 00 1A Response:0x19 01 00 24 09 03 15 51 47 00 00 00 00 00 00 F7 AA = 01: Current mode is active BB = 00: Mode type (e.g., Run) Timestamp: 24th September 2023, 15:51:47 End the Exercise Mode (e.g., Running Mode): Command:0x19 04 00 00 00 00 00 00 00 00 00 00 00 00 00 2C Response:0x19 01 00 24 09 03 15 51 47 00 00 00 00 00 00 F7 AA = 01: Success (mode ended) BB = 00: Mode type (e.g., Run) Timestamp: 24th September 2023, 15:51:47 Special Handling for Meditation Mode (BB = 6): Start Meditation Mode (Level 1, 10 minutes): Command:0x19 01 06 01 0A 00 00 00 00 00 00 00 00 00 00 2D BB = 06: Meditation mode CC = 01: Meditation level 1 DD = 0A: Duration of 10 minutes Pause Meditation Mode: Command:0x19 02 06 00 00 00 00 00 00 00 00 00 00 00 00 2E Response:0x19 01 00 24 09 03 15 51 47 00 00 00 00 00 00 F7 Error Response (if command is incorrect or conflicting): If an error occurs, such as attempting to start a new mode while another one is already active, the response will indicate an error: Response:0xA6 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC | JCV8 | 健康设置 Health Settings |  |
| 24 深 | 戒指心跳数据包（运动模式下的心率数据） Ring heart rate data packet (heart rate data in exercise mode) 圳市友宏科技有限公司版权 | 当通过APP打开戒指运动模式后 ，每秒会发送一个心跳数据包给 APP。 命令格式：18 HR S1 S2 S3 S4 K1 K2 K3 K4 T1 T2 T3 T4 D1 D2 D3 D4 00 00 00 HR: 0xFF: 说明是戒指结束运动模式 , 其它值是心率值， S1 S2 S3 S4: 步数。 K1 K2 K3 K4： 卡路里，浮点数。 T1 T2 T3 T4: 运动时间 D1 D2 D3 D4: 戒指计算的距离。 返回：18 FF XX 00 00 00 00 00 00 00 00 00 00 00 00 CRC 表示运动结束：XX是新增加的，如果是2，表示是自动检测到的结束 运动模式退出逻辑（需要在APP上对对应的弹窗提醒 ） 10分钟未达到80步---固件返回 18 aa 01（app增加弹窗：弹窗询问是否结束运动 ，点击否，运动继续，点击是，运动结束） 20分钟未达到80步---固件返回 18 aa 02（app增加弹窗：弹窗询问是否结束运动 ，点击否，运动继续，点击是，运动结束） 30分钟未达到80步---固件返回 18 ff 02（app增加弹窗：弹窗结束运动，点击确定按钮，直接退出运动） 所有 ！保密文件，请严格遵守保密协议，请勿外传给第三方！ | Command Format:0x18 HR S1 S2 S3 S4 K1 K2 K3 K4 T1 T2 T3 T4 D1 D2 D3 D4 00 00 00 HR: 0xFF indicates the ring has ended the exercise mode. Other values represent heart rate values. S1 S2 S3 S4: Step count. K1 K2 K3 K4: Calories (float value). T1 T2 T3 T4: Exercise duration. D1 D2 D3 D4: Distance calculated by the ring. Response: 18 FF XX 00 00 00 00 00 00 00 00 00 00 00 00 CRC Indicates the exercise has ended: XX is the newly added value. If it’s 2, it means the end was automatically detected. Exercise mode exit logic (requires corresponding reminder on the app): After 10 minutes with fewer than 80 steps, the firmware returns 18 aa 01 (the app shows a pop-up asking if the user wants to end the exercise. If "No" is selected, the exercise continues; if "Yes" is selected, the exercise ends). After 20 minutes with fewer than 80 steps, the firmware returns 18 aa 02 (the app shows a pop-up asking if the user wants to end the exercise. If "No" is selected, the exercise continues; if "Yes" is selected, the exercise ends). After 30 minutes with fewer than 80 steps, the firmware returns 18 ff 02 (the app shows a pop-up to end the exercise. If "Confirm" is clicked, the exercise exits immediately). | JCV8 | 健康设置 Health Settings |  |

| 深圳市友宏科技有限公司 （保密） Shenzhen Youhong Technology Co., Ltd. (Confidential) . |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 蓝牙穿戴产品 API 协议 产品名称：Smart screenless wristband 型号：JCV8 Bluetooth Wearable Product API Protocol. Product Name:Smart screenless wristband. Model: JCV8 |  |  |  |  |  |  |
| 序号 Serial Number | 功能描述 Function Description | 命令格式描述 | Command format description | 适配产品类型 Compatible product types | 分类 Category | 备注 Remarks |
| 25 | 获得各运动模式里的运动数据 （这里指运动模式里的数据 ，如跑步，骑自行车等） Obtain exercise data from various exercise modes (referring to data within the exercise modes, such as running, cycling, etc.) | 命令格式： 0x5C AA BB CC YY MM DD HH mm SS DD 00 00 00 00 CRC AA： 99:删除运动详细数据 00:读最近的运动数据 （读最近时间戳开始的 50条数据） YY MM DD HH mm SS:数据时间戳，发送00时携带时间戳可以只查询指定时间戳后的新数据 （第一次同步需要返回全部数据 ，00后的时间戳为历史数据中所存在的时间戳） 02:继续上次读的位置下一段数据 （读上一次读到的 50条数据的下一段 50条数据） 返回数据：（可变长指令） 0x5C ID1 ID2 YY MM DD HH mm SS TY HH tt tt SS SS PP PP KK KK KK KK DD DD DD DD METS YY MM DD HH mm SS:时间：年月日时分秒 TY :运动类型 0：Run， 1：Cycling - Badminton - Football - Tennis - Yoga - Meditation - Dance - Basketball - Walk - Workout 11、Cricket 12、Hiking - Aerobics - Ping-Pong - Rope Jump 16、Sit-ups 17、Volleyball HH :心率 Tt tt:运动时间 低位在前，高位在后 (以秒为单位, 高字节在后，低字节在前) SS SS;运动步数 低位在前，高位在后 (高字节在后，低字节在前) PP pp:运动速度 (大写PP为分, 小写pp为秒, 如PP=10, pp=50, 则表示走一公里所需时间为 10分钟50秒) KK KK KK KK:卡路里 低位在前，高位在后 浮点数遵循float ,单位为千卡。 DD DD DD DD: 距离 低位在前，高位在后 浮点数遵循float ,单位为千米。 RR RR RR RR: 运动卡路里，浮点数遵循float ,单位为千卡 ---注意这位是新增加的 ，只有当DD==1时才会返回例如：发送 0x5C 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x5C 00 00 24 09 03 10 36 47 00 8A 43 00 75 00 00 00 54 2A 0E 40 80 E3 6B 3D 5C FF 00 00：数据的ID编号¦ 24 09 03 10 36 47：24年9月3日10点36分47秒¦00：跑步¦8A：心率138¦43 00：67秒¦75 00：117步¦00 00：¦54 2A 0E 40：即0100 0000 0000 1110 0010 1010 0101 0100： | Command Format:0x5C AA BB CC 00 00 00 00 00 00 00 00 00 00 00 CRC AA: 99: Delete exercise detailed data 00: Read the latest exercise data (retrieve the latest 50 data points starting from the timestamp) YY MM DD HH mm SS: Timestamp; if 00 is sent, you can query for new data after the specified timestamp (for the first sync, all data will be returned, and the timestamp after 00 will be from the historical data). 02: Continue reading the next segment of data (retrieves the next 50 data points after the last read). Response data (variable-length command): 0x5C ID1 ID2 YY MM DD HH mm SS TY HH Tt tt tt SS SS PP PP KK KK KK KK DD DD DD DD YY MM DD HH mm SS: Date and time (Year, Month, Day, Hour, Minute, Second) TY: Exercise type 0: Run 1: Cycling 2: Badminton 3: Football 4: Tennis 5: Yoga 6: Meditation 7: Dance 8: Basketball 9: Walk 10: Workout 11: Cricket 12: Hiking 13: Aerobics 14: Ping-Pong 15: Rope Jump 16: Sit-ups 17: Volleyball HH: Heart rate Tt tt: Exercise time (low byte first, high byte later, in seconds) SS SS: Exercise steps (low byte first, high byte later) PP pp: Exercise speed (capital PP for minutes, lowercase pp for seconds, for example: PP=10, pp=50 means it takes 10 minutes and 50 seconds to walk 1 kilometer) KK KK KK KK: Calories (low byte first, high byte later, float value in kcal) DD DD DD DD: Distance (low byte first, high byte later, float value in kilometers) Example: Send: 0x5C 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0x5C 00 00 24 09 03 10 36 47 00 8A 43 00 75 00 00 00 54 2A 0E 40 80 E3 6B 3D 5C FF 00 00: Data ID 24 09 03 10 36 47: Timestamp (24th September, 3rd 10:36:47) 00: Exercise type (Running) 8A: Heart rate (138 bpm) 43 00: Exercise duration (67 seconds) 75 00: Steps taken (117 steps) 00 00: (Unused fields) 54 2A 0E 40: Calories and distance in float format (calories = 54.4 kcal, distance = 2.45 km) | JCV8 | 健康设置 Health Settings |  |
| 26 | 实时读取戒指温度 Real-time Temperature Reading of the Ring | 命令格式： 0x14 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 功能：实时读取戒指的温度命令回复： 校验正确且执行OK返回：0x14 AA BB CC DD EE FF GG HH II JJ 00 00 00 00 CRC AA BB： 戒指温度（取目前3个NTC中实时的最高的温度值 ），16进制小端 CC DD： 戒指温度，10进制，保留一位小数，如02 64，则解析为26.4℃ 例如：发送 0x14 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 返回 0x14 48 01 03 28 45 01 48 01 41 01 00 00 00 00 CRC 48 01：温度32.8℃¦03 28：温度十进制显示，即32.8℃¦45 01：32.5℃ | Command Format:0x14 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Function: Real-time reading of the ring’s temperature. Command Response: If the checksum is correct and the execution is successful, the response is: 0x14 AA BB CC DD EE FF GG HH II JJ 00 00 00 00 CRC AA BB: Ring temperature (the highest temperature value from the current 3 NTCs, in hexadecimal little-endian format). CC DD: Ring temperature, in decimal format with one decimal place. For example, if 02 64, it should be interpreted as 26.4℃. EE FF GG HH II JJ: Temperature data from the 3 NTCs, where every two bytes represent the temperature data for one NTC. For example, EE FF represents the temperature data of one NTC, in little-endian hexadecimal. Example: Sending: 0x14 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC Response: 0x14 48 01 03 28 45 01 48 01 41 01 00 00 00 00 CRC 48 01: Temperature is 32.8℃. 03 28: Temperature in decimal format, i.e., 32.8℃. 45 01: Temperature of 32.5 ℃. | JCV8 | 原始数据 Raw Data |  |
|  | 获得睡眠详细数据+活动量 | 命令格式： 0x6B AA BB CC 00 00 00 00 00 00 00 00 00 00 00 CRC AA: 99: 删除睡眠详细数据 , 0:读最近的睡眠详细数据 。 1：读指定位置的睡眠详细数据 。 2：继续上次读的位置下一段数据 。 返回数据。（可变长指令） 0JCV8B ID1 ID2 YY MM DD HH mm SS LEN SD1 SD2 SD3 ... SD120 最小单位是130 个字节。可根据实际最大发送包的长度动态控制一个 Bluetooth 蓝牙数据包包含多少天的总数据 。 ID1 ID2：读数据编号，高位在后。。 YY MM DD HH mm SS：表示数据的日期 ：年月日时分秒 LEN:长度表示睡眠数据的长度 。 SD1 SD2 SD3 ... SD120: 最多表示2小时的睡眠数据和活动量 。 前60个字节，每个字节表示 2分钟的睡眠级别 ，高半字节表示一分钟 ，低半字节表示第二分钟 ，后面60个字节，每个字节表示 2分钟的活动量。 |  |  |  |  |
|  | 实时温度 | 命令格式： 0x14 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 0x14 AA BB CC DD T1 T2 T3 T4 T5 T6 00 00 00 00 CRC AA BB:最终温度值，使用三个通道的最大值 ，高位在后，保留一位小数 T1 T2: 温度值，高位在后，保留一位小数 |  |  |  |  |
|  | 血糖测试 | 命令格式：0x78 AA BB 00 00 00 00 00 00 00 00 00 00 00 00 CRC 描述：AA： 1：开始血糖测试 2：测试成功 BB：测试结果：1：低 2：中 3：高 0：测试失败 3：停止信号采集 4：进度更新 BB：测试进度，0~100 5：退出当前血糖测量界面 ，跳转到主界面命令回复： 校验正确且执行OK返回：0x78 AA BB 00 00 00 00 00 00 00 00 00 00 00 00 CRC 描述： AA：0：成功 2：电池电量过低，少于10%，只有开始测试血糖时才有效 3：在跑步模式，不能开始测试 BB：1：开始检测 2：测试成功 3：停止信号采集 4：进度更新 5：退出血糖测试界面 检验错误或执行Fail返回：0xA8 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 9、血糖原始数据返回 0x3A ID1 ID2 DATA1 DATA2 DATA3 DATA4 ... DATA200 描述：ID1 ID2：流水号，高位在前 DATA1~DATA200 ：血糖数据（PPG数据）4个字节表示一个PPG数据，高位在前 |  |  |  |  |
|  | 设置戒指基本参数 | 命令格式： 0x03 AA BB CC DD EE FF GG HH II JJ KK LL MM NN CRC 功能：向戒指写入基本参数描述： EE:运动模式是否闪灯 0x81 ：使能. 0X80:关闭 燃脂区间闪烁蓝色慢闪 ， 有氧闪烁绿色慢闪 ，无氧闪烁红慢闪， FF:运动模式心率过高是否闪灯 0x81：使能. 0X80: 关闭 心率过高红色快闪 NN： 全天心率功能 0x81 ：使能. 0X80:关闭 命令回复： 校验正确且执行OK返回：0x03 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 校验错误或执行Fail返回：0x83 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |  |
|  | 18. 设置闹钟（可变长指令）： | 命令格式： 0x23 XX AA BB CC DD EE FF GG T1 T2 ... T30 可变长是需要根据手机实际的最大长度选择指令 。如本条指令单个长度是 37个字节，如果手机最大长度是 200个字节。那一条指令可以发送 200/37=5.4, 取整后是5，也就是说一条指令就可以把 5个闹钟设置好。 指令实例结构如下。 0x23 05 00 BB CC DD EE FF GG T1 T2 ... T30 0x23 05 01 BB CC DD EE FF GG T1 T2 ... T30 0x23 05 02 BB CC DD EE FF GG T1 T2 ... T30 0x23 05 03 BB CC DD EE FF GG T1 T2 ... T30 0x23 05 04 BB CC DD EE FF GG T1 T2 ... T30 0x23 FF(说明是结束，这个时候会保存数据 ) 功能：设置闹钟功能。描述： XX 总共使用的闹钟个数 。 AA范围是0到9闹钟编号,（取值为0 to 9）,可设置10个闹钟。 - 0为Disable; 1为Enable - 闹钟类型， 1闹钟，2吃药提示，3：喝水提示。 - 为闹钟时间的小时部分 （24小时制），BCD码格式（如23点CC = 0x23）。 - 为闹钟时间的分钟部分 ，BCD码格式（如59分CC = 0x59 ）。 - 星期使能。Bit0 = 0 表示星期天不使能，Bit0 = 1表示星期天使能。 Bit1 = 0 表示星期一不使能 ，Bit1 = 1表示星期一使能。 Bit2 = 0 表示星期二不使能 ，Bit2 = 1表示星期二使能。 Bit3 = 0 表示星期三不使能 ，Bit3 = 1表示星期三使能。 Bit4 = 0 表示星期四不使能 ，Bit4 = 1表示星期四使能。 Bit5 = 0 表示星期五不使能 ，Bit5 = 1表示星期五使能。 Bit6 = 0 表示星期六不使能 ，Bit6 = 1表示星期六使能。 GG: 长度，0到30，表示后面文本显示的长度 T1 T2 ... T30，文本的内容，不管前面长度是多少 ，没用的位置用 0表示，内容是ASCII编码。 |  |  |  |  |
|  | 28. 获得闹钟数据 | 28. 获得闹钟数据 命令格式： 0x24 AA BB CC 00 00 00 00 00 00 00 00 00 00 00 CRC AA: 99: 删除闹钟数据 , 0:读闹钟数据。 返回数据。（可变长指令） 0x24 ID1 ID2 AA BB CC DD EE FF T1 T2 ... T30 描述： ID1 ID2：读数据编号，高位在后。 AA范围是0到4闹钟编号,（取值为0 to 4）,可设置5个闹钟。 BB闹钟类型，0为Disable; 1为Enable闹钟，2吃药提示，3：喝水提示。 CC 为闹钟时间的小时部分 （24小时制），BCD码格式（如23点CC = 0x23）。 DD 为闹钟时间的分钟部分 ，BCD码格式（如59分CC = 0x59 ）。 EE 星期使能。Bit0 = 0 表示星期天不使能，Bit0 = 1表示星期天使能。 Bit1 = 0 表示星期一不使能 ，Bit1 = 1表示星期一使能。 Bit2 = 0 表示星期二不使能 ，Bit2 = 1表示星期二使能。 Bit3 = 0 表示星期三不使能 ，Bit3 = 1表示星期三使能。 Bit4 = 0 表示星期四不使能 ，Bit4 = 1表示星期四使能。 Bit5 = 0 表示星期五不使能 ，Bit5 = 1表示星期五使能。 Bit6 = 0 表示星期六不使能 ，Bit6 = 1表示星期六使能。 FF: 长度，0到30，表示后面文本显示的长度 T1 T2 ... T30，文本的内容，不管前面长度是多少 ，没用的位置用 0表示，内容是ASCII编码。 |  |  |  |  |
|  | ECG原始数据 | ECG原始数据 命令格式： 0x7 AA 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC AA: 1， 使能ECG原始数据发送 0：关闭ECG原始数据发送 当使能ECG原始数据之后，需要配合28 04 01打开ECG功能，然后有ECG数据时，会自动返回给APP，格式如下 07 AA D1 D2 D3 ... DX AA:流水号 D1 D2 D3 ... DX,:ECG原始数据。三个一组。高位在后。 |  |  |  |  |
|  | 获得PPI数据 | 获得PPI数据 命令格式： 0x63 AA BB CC 00 00 00 00 00 00 00 00 00 00 00 CRC AA: 99: 删除PPI数据 , 0:读最近的PPI数据。 1：读指定位置的PPI数据。 2：继续上次读的位置下一段数据 。 返回数据。（可变长指令） 0JCV83 ID1 ID2 YY MM DD HH mm CC ID P1 P2 … P112 ID1 ID2：读数据编号，高位在后。 YY MM DD HH mm SS：表示数据的日期 ：年月日时分秒,这个是PPI保存时间点，不是开始时间点 CC:本组数据的条数 ID:本条数据在本组数据的 ID号。 |  |  |  |  |
|  | 获得戒指基本参数 | 命令格式： 0x04 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 命令回复： 校验正确且执行OK返回：0x04 AA BB CC DD EE FF GG HH II JJ KK LL MM NN CRC EE:运动模式是否闪灯 燃脂区间闪烁蓝色慢闪 ， 有氧闪烁绿色慢闪，无氧闪烁红慢闪 ， FF:运动模式心率过高是否闪灯心率过高红色快闪 NN： 全天心率功能 0x81 ：使能. 0X80:关闭 校验错误或执行Fail返回：0x83 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC |  |  |  |  |
|  | SOS | 设备长按之后发送。 0xFE 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC APP收到SOS指令之后，需要回复 0xFE 00 00 00 00 00 00 00 00 00 00 00 00 00 00 CRC 如果蓝牙断开或 3秒没有收到APP回复，设备显示失败灯效 。收到回复显示成功灯效果 。 |  |  |  |  |

| 深圳市友宏科技有限公司 （保密） Shenzhen Youhong Technology Co., Ltd. (Confidential) . |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 蓝牙穿戴产品 API 协议 产品名称：Smart screenless wristband 型号：JCV8 Bluetooth Wearable Product API Protocol. Product Name:Smart screenless wristband. Model: JCV8 |  |  |  |  |  |  |
| 序号 Serial Number | 功能描述 Function Description | 命令格式描述 | Command format description | 适配产品类型 Compatible product types | 分类 Category | 备注 Remarks |